TABLE OF CONTENTS

1	HDD Overview	3

2	Safety Board Overview	3

3	Input and Output Signals	3

4	Analog Velocity Compensation Circuit	6

5	Fault Detection & Control Inputs	6

5.1	Adjustable, Analog Faults	10

6	Miscellaneous Circuits	10

6.1	Relays	10

6.2	Watchdog Timer	10

6.3	CPLD Latch Clock	10

7	Known Issues / Recommendations	10

7.1	Comparator Output Logic Level Mismatch	10

7.2	Faults Detected by TCS3 With No Latched Faults	11

7.3	Overcurrent and Overspeed Comparators Trip Points Not Symmetrical	11

7.4	Relays	11

7.5	Miscellaneous	12

TABLE OF FIGURES & REPORT TABLES

Figure 1	Simplified analog velocity circuitry block diagram	6

Figure 2	Completer CPLD logic schematic	7

Figure 3	Zoom in of override faults and clock circuitry	8

Figure 4	Zoom in of CPLD output signals.	9

Figure 5	Redundant Tel_Enable Logic	9

Table 1	Connector P1 – Main Signal Inputs	4

Table 2	Connector P2 – Main Signal Outputs	5

Table 3	Connector P3 - Power	5

Table 4	Connector P4 – Spare Differential Line Receiver	5

Table 5	Connector JP1 – JTAG	6

Table 6	CPLD output signals	7

Table 7	Adjustable analog faults	10

# HDD Overview

This hardware description document (HDD) explains the functions and the input and output signals of the Safety Board.  The Safety Board revision at the time of this document creation was Rev C.

Also included is a section on known issues, severity of the issues, and how they may be resolved.  Critical issues have been fixed.  However, if another layout is ever done, there may be better ways to fix the issue in a new layout vs. the simple “green wire” modifications.  For known issues that weren’t severe and were not fixed, they are documented for future layout.

# Safety Board Overview

The Safety Board has two main purposes:

- To detect faults or inputs and disable the telescope.
This protects personnel and the telescope.  Hence, the name “Safety Board”.

- Contains an analog velocity circuit for TCS3 which ultimately provides the amplifier command.

# Input and Output Signals

3.1 Connector P1 – Main Signal Inputs

| Pin | Signal Name | I/O | Type | Level | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | East Drive+ | IN | Analog | +/- 25V Differential | East tachometer. Negative terminal is reference. Scale factor is approximately 8.764 mV/(arcsec/s). 2000arcsec/s = ~17.5V. |
| 2 | East Drive- | IN |  |  |  |
| 3 | AGND | - | GND | GND |  |
| 4 | West Drive- | IN | Analog | +/- 25V Differential | West tachometer. Negative terminal is reference. Scale factor is approximately 8.764 mV/(arcsec/s). 2000arcsec/s = ~17.5V. |
| 5 | West Driv+ | IN |  |  |  |
| 6 | AGND | - | GND | GND |  |
| 7 | South Drive+ | IN | Analog | +/- 25V Differential | South tachometer. Negative terminal is reference. Scale factor is approximately 8.764 mV/(arcsec/s). 2000arcsec/s = ~17.5V. |
| 8 | South Drive- | IN |  |  |  |
| 9 | AGND | - | GND | GND |  |
| 10 | North Drive+ | IN | Analog | +/- 25V Differential | North tachometer. Negative terminal is reference. Scale factor is approximately 8.764 mV/(arcsec/s). 2000arcsec/s = ~17.5V. |
| 11 | North Drive- | IN |  |  |  |
| 12 | AGND | - | GND | GND |  |
| 13 | East Vel In | IN | Analog | 0 - 10V | East motor PMAC velocity DAC output. |
| 14 | West Vel In | IN | Analog | 0 - 10V | West motor PMAC velocity DAC output. |
| 15 | South Vel In | IN | Analog | 0 - 10V | South motor PMAC velocity DAC output. |
| 16 | North Vel In | IN | Analog | 0 - 10V | North motor PMAC velocity DAC output. |
| 17 | HA Vel Feedback | OUT | Analog | 0 – 5V | Average of East and West tachometers. 596.83 arcscec/V |
| 18 | Dec Vel Feedback | OUT | Analog | 0 – 5V | Average of North and South tachometers. 596.83 arcscec/V |
| 19 | Mtr Cntr Err | IN | Digital | 0 – 5V | Driven by PC Parallel port. Not used. Set to 0V. |
| 20 | TCS Lockout | IN | Digital | 0 – 5V | Driven by PC Parallel port. Not used. Set to 0V. |
| 21 | Brake En In | IN | Digital | 0 – 5V | Driven by PC Parallel port. 0V=brake, 5V= brake disabled |
| 22 | Watchdog In | IN | Digital | 0 – 5V | Driven by PC Parallel port. 5V=watchdog pulse |
| 23 | Reset | IN | Digital | 0 – 5V | Driven by PC Parallel port. 0V=reset, 5V=not in reset mode |
| 24 | West Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 25 | East Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 26 | North Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 27 | South Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 28 | Dome1 Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 29 | Dome2 Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 30 | Dome3 Current | IN | Analog | 0 – 10V | From NC307 amp. Scale factor is 11.66A/V. |
| 31 | Horizon Stop | IN | Switch | GND or open | Switch, Dec Emergency Stop, GND=OK, open=stop |
| 32 | HA Stop W | IN | Switch | GND or open | Switch, HA W Stop, GND=OK, open=stop |
| 33 | HA Stop E | IN | Switch | GND or open | Switch, HA E Stop, GND=OK, open=stop |
| 34 | HA Emerg W | IN | Switch | GND or open | Switch, HA W Emergency Stop, GND=OK, open=stop |
| 35 | HA Emerg E | IN | Switch | GND or open | Switch, HA E Emergency Stop, GND=OK, open=stop |
| 36 | Dec Stop N | IN | Switch | GND or open | Switch, Dec N Stop, GND=OK, open=stop |
| 37 | Dec Stop S | IN | Switch | GND or open | Switch, Dec S Stop, GND=OK, open=stop |
| 38 | Dec Emerg N | IN | Switch | GND or open | Switch, Dec N Emergency Stop, GND=OK, open=stop |
| 39 | Dec Emerg S | IN | Switch | GND or open | Switch, Dec S Emergency Stop, GND=OK, open=stop |
| 40 | Emerg Stop | IN | Switch | 5V or open | Switch, Emergency Stop. Open=STOP, 5V=do not stop |
| 41 | TOP Dome Cntl Software | IN | Switch | 5V or open | Switch, Dome under Software Control, open=Disable, 5V=Enable |
| 42 | TOP Tel Enable Switch | IN | Switch | 5V or open | Switch, Telescope Enable, open=Disable, 5V=Enable |
| 43 | TOP Lm Overrride Switch | IN | Switch | 5V or open | Switch, Limit Override, open=Override Inactive, 5V=Override Mode |
| 44 | HP Stop | IN | Switch | GND or open | Switch, Hand Paddle Stop button, GND=OK, Open=STOP |
| 45 | Spare In 1 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |
| 46 | Spare In 2 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |
| 47 | Spare In 3 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |
| 48 | Spare In 4 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |
| 49 | Spare In 5 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |
| 50 | Spare In 6 | IN | Switch | N/A | Not used, pulled up to VCC via 1kΩ |

Table 1	Connector P1 – Main Signal Inputs

3.2 Connector P2 – Main Signal Outputs

| Pin | Signal Name | I/O | Type | Level | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 24V | IN | Power | 24V | Power for relay output side. |
| 2 | Brake Enable Relay | OUT | Analog | Open or 24V | Relay output for brake. Open=enable, 24V=disable |
| 3 | East Tach Out | OUT | Analog | 0 - 5.5 V | Buffered, filter East tachometer. ~1.845 mV/(arcsec/s) |
| 4 | Tel Amp | OUT | Analog | NA – relay output | Relay connection for NC307 amplifier enable. When relays activated on Safety Board, closed contacts enable amplifiers. Relay connection. |
| 5 | Tel Amp Return | OUT |  |  |  |
| 6 | Dome Amp | OUT | Analog | NA – relay output | Relay connection for NC307 amplifier enable. When relays activated on Safety Board, closed contacts enable amplifiers. |
| 7 | Dome Amp Return | OUT |  |  |  |
| 8 | West Tach Out | OUT | Analog | 0 - 5.5 V | Buffered, filter West tachometer. ~1.845 mV/(arcsec/s) |
| 9 | South Tach Out | OUT | Analog | 0 -5.5 V | Buffered, filter South tachometer. ~1.845 mV/(arcsec/s) |
| 10 | East Vel Out | OUT | Analog |  | East amplifier command voltage. 10.5A/V |
| 11 | AGND | - | GND | GND |  |
| 12 | West Vel Out | OUT | Analog |  | West amplifier command voltage. 10.5A/V |
| 13 | AGND | - | GND | GND |  |
| 14 | South Vel Out | OUT | Analog |  | South amplifier command voltage. 10.5A/V |
| 15 | AGND | - | GND | GND |  |
| 16 | North Vel Out | OUT | Analog | - | North amplifier command voltage. 10.5A/V |
| 17 | AGND | - | GND | GND |  |
| 18 | HA OS Latch | OUT | Digital | 0 – 5V | HA overspeed fault latched. 0=no fault, 5V=fault. |
| 19 | Dec OS Latch | OUT | Digital | 0 – 5V | HA overspeed fault latched. 0=no fault, 5V=fault. |
| 20 | West OC Latch | OUT | Digital | 0 – 5V | West motor overcurrent fault latched. 0=no fault, 5V=fault. |
| 21 | East OC Latch | OUT | Digital | 0 – 5V | East motor overcurrent fault latched. 0=no fault, 5V=fault. |
| 22 | North OC Latch | OUT | Digital | 0 – 5V | North motor overcurrent fault latched. 0=no fault, 5V=fault. |
| 23 | South OC Latch | OUT | Digital | 0 – 5V | South motor overcurrent fault latched. 0=no fault, 5V=fault. |
| 24 | Dome1 OC Latch | OUT | Digital | 0 – 5V | Dome motor1 overcurrent fault latched. 0=no fault, 5V=fault. |
| 25 | Dome2 OC Latch | OUT | Digital | 0 – 5V | Dome motor2 overcurrent fault latched. 0=no fault, 5V=fault. |
| 26 | Dome3 OC Latch | OUT | Digital | 0 – 5V | Dome motor3 overcurrent fault latched. 0=no fault, 5V=fault. |
| 27 | Emerg Stop Latch | OUT | Digital | 0 – 5V | Emergency Stop latched. 0=no fault, 5V=fault. |
| 28 | HP Stop Latch | OUT | Digital | 0 – 5V | Hand Paddle Stop latched. 0=no fault, 5V=fault. |
| 29 | Mtr Cntr Err Latch | OUT | Digital | 0 – 5V | Not used |
| 30 | TCS Lockout Latch | OUT | Digital | 0 – 5V | Not used |
| 31 | HA Stop W Latch | OUT | Digital | 0 – 5V | West travel stop 0=no fault, 5V=fault. |
| 32 | HA Stop E Latch | OUT | Digital | 0 – 5V | East travel stop. 0V=no fault, 5V=fault |
| 33 | HA Emerg W Latch | OUT | Digital | 0 – 5V | West travel emergency stop. 0V=no fault, 5V=fault |
| 34 | HA Emerg E Latch | OUT | Digital | 0 – 5V | East travel emergency stop. 0V=no fault, 5V=fault |
| 35 | Dec Stop N Latch | OUT | Digital | 0 – 5V | North travel stop 0=no fault, 5V=fault. |
| 36 | Dec Stop S Latch | OUT | Digital | 0 – 5V | South travel stop 0=no fault, 5V=fault. |
| 37 | Dec Emerg N Latch | OUT | Digital | 0 – 5V | North travel emergency stop. 0V=no fault, 5V=fault |
| 38 | Dec Emerg S Latch | OUT | Digital | 0 – 5V | South travel emergency stop. 0V=no fault, 5V=fault |
| 39 | Horizon Stop Latch | OUT | Digital | 0 – 5V | Horizon stop. 0V=no fault, 5V=fault |
| 40 | Watchdog Timer Latch | OUT | Digital | 0 – 5V | No watchdog timer latched. 0=no fault, 5V=timer fault. |
| 41 | Spare Latch Out 1 |  | Digital | 0 – 5V | Not used |
| 42 | Spare Latch Out 2 | OUT | Digital | 0 – 5V | Not used |
| 43 | Spare Latch Out 3 | OUT | Digital | 0 – 5V | Not used |
| 44 | Spare Latch Out 4 | OUT | Digital | 0 – 5V | Not used |
| 45 | Spare Latch Out 5 | OUT | Digital | 0 – 5V | Not used |
| 46 | Spare Latch Out 6 | OUT | Digital | 0 – 5V | Not used |
| 47 | North Tach Out | OUT | Digital | 0 - 5.5 V | Buffered, filter South tachometer. ~1.845 mV/(arcsec/s) |
| 48 | TOP Dome Cntl Handpaddle | OUT | Digital | 0 – 5V | Dome Control switch. 0V=not handpaddle 5V=handpaddle control |
| 49 | TOP TCS Enable | OUT | Digital | 0 – 5V | TO Panel TCS LED. 0V=OFF, 5V=ON |
| 50 | TOP Brake Enable Out | OUT | Digital | 0 – 5V | TO Panel Brake LED. 0V=OFF, 5V=ON |

Table 2	Connector P2 – Main Signal Outputs

3.3 Connector P3 - Power

| Pin | Signal Name | I/O | Type | Level | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | VCC | IN | Power | +5V | +5V Digital Power |
| 2 | DGND | - | GND | GND | Return for digital power. |
| 3 | +15V | IN | Power | +15V | +15V for op-amps, comparators, etc. |
| 4 | AGND | - | GND | GND | Return for analog power. |
| 5 | -15V | IN | Power | -15V | -15V for op-amps, comparators, etc. |

Table 3	Connector P3 - Power

3.3 Connector P4 – Spare Differential Line Receiver

This connector consists of the inputs and outputs of a DS26LS32AC differential line receiver.

| Pin | Signal Name | I/O | Type | Level | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | OUT A | OUT | Digital | 0 – 5V | Spare output. |
| 2 | IN A+ | IN | Differential | -7 to +7V | Spare input. |
| 3 | OUT B | OUT | Digital | 0 – 5V | Spare output. |
| 4 | IN A- | IN | Differential | -7 to +7V | Spare input. |
| 5 | OUT C | OUT | Digital | 0 – 5V | Spare output. |
| 6 | IN B+ | IN | Differential | -7 to +7V | Spare input. |
| 7 | OUT D | OUT | Digital | 0 – 5V | Spare output. |
| 8 | IN B- | IN | Differential | -7 to +7V | Spare input. |
| 9 | NC | - | - | - |  |
| 10 | IN C+ | IN | Differential | -7 to +7V | Spare input. |
| 11 | NC | - | - | - |  |
| 12 | IN C- | IN | Differential |  | Spare input. |
| 13 | VCC | OUT | Power | +5V | Output power, +5V. |
| 14 | IN D+ | IN | Differential | -7 to +7V | Spare input. |
| 15 | DNGD | - | GND | GND |  |
| 16 | IN D- | IN | Differential | -7 to +7V | Spare input. |

Table 4	Connector P4 – Spare Differential Line Receiver

3.4 Connector JP1 – JTAG

| Pin | Signal Name | I/O | Type | Level | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DGND | - | GND | GND |  |
| 2 | VCC | OUT | Power | +5V | +5V used for JTAG connector power out. |
| 3 | DGND | - | GND | GND |  |
| 4 | TMS | IN | Digital | 0 – 3.3V | JTAG Test Mode Select |
| 5 | DGND | - | GND | GND |  |
| 6 | TCK | IN | Digital | 0 – 3.3V | JTAG Test Clock |
| 7 | DGND | - | GND | GND |  |
| 8 | TDO | OUT | Digital | 0 – 3.3V | JTAG Test Data Out |
| 9 | DGND | - | GND | GND |  |
| 10 | TDI | IN | Digital | 0 – 3.3V | JTAG Test Data In |
| 11 | DGND | - | GND | GND |  |
| 12 | NC | - | - | - |  |
| 13 | DGND | - | GND | GND |  |
| 14 | NC | - | - | - |  |

Table 5	Connector JP1 – JTAG

# Analog Velocity Compensation Circuit

The analog velocity circuit can best be described with a very simple block diagram.  This diagram does not show any filtering or any frequency dependent characteristics.  The DEC axis is shown as an example.  There is no adjustment for this circuitry.

Figure 1	Simplified analog velocity circuitry block diagram

Essentially the velocity circuitry does 3 main things

- Buffers and divides each tachometer input

- Averages the tachometers together for one value to be used by the PMAC servo

- Adds some frequency dependent compensation to the velocity command from the PMAC

# Fault Detection & Control Inputs

The Safety Board detects various faults and accepts multiple control signals.  These faults and control signals are used as inputs to logic programmed into a CPLD.  The combination of those signals and faults determines the state of the four output signals.

| Signal Name | Connector | Pin | Description |
| --- | --- | --- | --- |
| Dome Enable | NA | On Board | Sinks current for (enables) Dome Amp Relay. 0V=ON, 5V=OFF |
| TOP TCS Enable | P2 | 49 | Powers TO Panel Brake LED. 0V=OFF, 5V=ON |
| Tel Enable | NA | On Board | Sinks current for (enables) Brake Enable and Telescope Amp Relays. 0V=disable brakes, enable tel amp, 5V=enable brakes, disable tel amp |
| TOP Brake Enable Out | P2 | 50 | Powers TO Panel Brake LED. 0V=OFF, 5V=ON |

Table 6	CPLD output signals

The logic programmed into the CPLD can be seen in a schematic form in “T3-2071-Safety_CPLD_Logic.pdf”.  This schematic is shown below, but may be difficult to read.

Figure 2	Completer CPLD logic schematic

Figure 3	Zoom in of override faults and clock circuitry

From the zoomed in portion of the logic in Figure 3, it can be seen that ten faults can be overridden using TOP Limit Override and they consist of the HA and DEC axis stops, the Horizon Stop, and spare1.  Notice XLXI_54 and XLXI_55.  These D flip-flops load D into Q on a low to high transition.  It takes two clock cycles from when a fault appears (“1”) on the input of XLXI_54 to change the output of XLXI_51.  The output of XLXI_51 latches all fault inputs and is the final fault signal that puts the CPLD output signals into fault states.  This means that a fault must be present longer than a clock cycle, minimum, or 133 ms (1/7.5Hz).

Fault Signal

Figure 4	Zoom in of CPLD output signals.

Looking at Figure 4, a Reset state of  “0” or a Fault Signal state of “0” will cause all of the CPLD’s outputs to go into fault states.  Fault states put all outputs to “1” except “TOP TCS Enable”, which goes to “0”.

If there is no fault or reset and if “TOP_DomeCntl_Software” or “Top_DomeCntl_Handpaddle” is a logic “1”, then the “Dome Enable” signal will go low and activate a relay enabling the dome amplifiers.

If there is no fault or reset and if “TOP Tel Enable Switch” or “Brake En In” go to a logic “0”, then “Tel Enable” will go to logic “1” and turn off the relay that enables the amplifier and will also turn off the relay that disables the brakes (brakes now ON).  The “TOP_Brak_Enable_Out” will become a logic “1” and power the Brake LED on the TO panel.

Figure 5	Redundant Tel_Enable Logic

The redundant Tel_Enable signal is controlled by 3 signals.  The Reset and Tel_Amp_Brake signals are simply ANDed together.  The last signal is the the ORing of two watchdog timers.  There are two clock signals – Clk_In and RAW_WATCHDOG_CLK.  The Clk_In is the local oscillator on the Safety Board and the RAW_WATCHDOG_CLK is the watchdog clock from the parallel port of the TCS3 PC.  These two clocks are used to form two watchdog timers that essentially check each other.  If either clock is removed, an error will occur.  The final output signal is logic high, which is opposite of the active logic low signal on the main Tel_Enable signal.  Having opposite logic enable signals reduces that chance of a fault state not disabling the telescope.

## Adjustable, Analog Faults

There are overspeed and overcurrent faults that are adjustable via potentiometers.  The table below lists the faults that are adjustable.

| Fault Name | Type | Description |
| --- | --- | --- |
| HA_Overspeed | Overspeed | HA Axis overspeed. |
| Dec_Overspeed | Overspeed | DEC Axis overspeed. |
| West_Overcurrent | Overcurrent | West motor overcurrent. |
| East_Overcurrent | Overcurrent | East motor overcurrent. |
| North_Overcurrent | Overcurrent | North motor overcurrent. |
| South_Overcurrent | Overcurrent | South motor overcurrent. |
| Dome1_Overcurrent | Overcurrent | Dome motor1 overcurrent. |
| Dome2_Overcurrent | Overcurrent | Dome motor2 overcurrent. |
| Dome3_Overcurrent | Overcurrent | Dome motor3 overcurrent. |

Table 7	Adjustable analog faults

# Miscellaneous Circuits

## Relays

There are three relays.  Two realys enable/disbale the amplifiers for the telescope and dome.  The other relay enables/disables the telescope brakes.

## Watchdog Timer

The watchdog timer receives a pulse from the TCS3 PC via a parallel port.  It has a time delay of 600 ms.  A high to low transition resets the timer.  If another high to low transition does not occur within 600 ms, the watchdog timer with go to a logic low (reset) state.

## CPLD Latch Clock

A 7.5 Hz clock is used as an input into the CPLD for the transparent latches contained in the CPLD logic.

# Known Issues / Recommendations

## Comparator Output Logic Level Mismatch

Severity: High
Status: Board Modified, Problem Fixed

Issue
The comparators had an output logic of -15V and +5V (through a pullup resistor).  The reason that the logic level was -15V was because the comparators had to sense positive and negative values and therefore had +/-15V supplies.  This was perfectly fine.  However, the outputs of the comparators were directly connected to the CPLD.  It required logic of 0 or +5V.  The -15V on the CPLD forward biased the protection diodes on the CPLD pin inputs.  This in turn put the comparators in a current limited state and caused the CPLD to dissipate the power of the diode drop multiplied by the current pulled through the comparator.

Fix
The CPLD can withstand -0.5V minimum, which is right below the forward diode drop.  The easiest solution to implement on the existing boards was to create a -0.7 V “power supply”.  This supply was nothing more than a resistor and a diode clamp since a large amount of current was not necessary.  With an actual supply of around -0.65V, and a drop of around 150mV for the comparator output, the input signal was very close to -0.5V.  Since error signals are only present for a short time, this should not stress the CPLD.  Changing the supply also required lowering the reference voltages from +/-1V to +/-0.44 V.

Relayout Suggestion
If relaying out the board, using -15V for the comparators with some type of level shifter buffer after the output would be preferable.

## Faults Detected by TCS3 With No Latched Faults

Severity: Very Low, Informative
Status: No changes to Safety Board

Issue
The CPLD uses transparent latches that latch when the 7.5 Hz external clock transitions from high to low.  A transparent latch allows the input to pass through to the output while the clock is high and latches the value on the input to the output when the clock goes high to low.  Furthermore, the fault signal must remain for an entire clock cycle for the safety board to enter a fault state.  If a fault condition occurs for a short period of time, less than 133ms (1/7.5Hz), the TCS3 OPTO22 modules which monitor all fault outputs will report a fault, but the Safety Board will not enter a  fault state.  This acts as a filter that will filter out small glitches.  All fault inputs to the Safety Board will be slow, long term faults much greater than 133ms.  So, if errors “blip” on the TCS3 control screen, this is why.

## Overcurrent and Overspeed Comparators Trip Points Not Symmetrical

Severity: Medium 
Status: Overspeed circuits modified, Overcurrent unmodified

Issue
The comparator circuits for the overspeed and overcurrent are not driven be low impedance drivers.  Instead, they are driven by a resistor divider.  Since the negative trip point is not high impedance, loading on the resistor divider occurs.  This translates into unsymmetrical trip points, that is, the positive and negative magnitudes are not the same.  For the overcurrent comparators, this is actually acceptable.  The motors are only driven in one direction, so current only flows in one direction.  Therefore, requiring a larger (or smaller) value in the opposite direction doesn’t matter.  It still trips on catastrophic (i.e. short) overcurrent conditions.

Fix
The Overspeed comparators need to be more symmetrical.  The dividers were reduced in value from  a total of 60kΩ to 2kΩ.  This reduced the loading error and made the outputs more symmetrical, but not perfectly.

Relayout Suggestion
If relaying out the board, place a buffer such as an op-amp follower with a gain of one in between the voltage divider and the comparator circuit.

## Relays

Severity: Medium
Status: Board modified, CPLD logic modified

Issue
The Brake Enable and Telescope Amplifier relays are driven by the same active low signal.  If a short occurred on this signal, then the telescope would have no brakes and the amplifiers would be enabled.  This is the normal condition when the servo is in operation.  However, if the servo exits due to an error or if the Safety Board (which ultimately sends the command to the amplifier) has some fault which results in an unexpected command, then the telescope will not be under control.

Fix
An NPN transistor was added in series with the current active low output signal.  The NPN transistor is driven with a logic high signal from the CPLD.  Two signals with opposite logic greatly reduce the probability of a fault or failure occurring that does not disable the amplifiers and turn of the brakes.  Furthermore, the new redundant signal is controlled by additional, unrelated input.  In this case, watchdog timers monitoring the two clock signals were used.

Relayout Suggestion
If relaying out the board, two relays in series for the brake signal, each controlled by  completely separate signal could be implemented.  One step beyond that would be having one of the relays controlled by a pulse.  This pulse could be of a fixed frequency, filtered, and then used with a diode and capacitor to create a charge pump.

## Miscellaneous

Severity: Low
Status: No changes to Safety Board

Relayout Suggestion
If relaying out the board, there are some minor additions that may be preferable.  They are not absolutely necessary, but may provide a more robust design.

1) Schmitt Triggers on switch inputs after the filter provide clean digital signals.

2) When driving off board signals from and CPLD or FPGA, a buffer may be good idea to protect the CPLD against shorts, and to provide the current to external loads such as LEDs.

3) Bias current canceling resistors on grounded op-amp terminals are desirable.