T3 Project Requirements

1. General T3 Requirements

1.1 Overall goals of T3 project is to replace the following equipment:

- LSI II/23 Computer system.

- CCS and MCC Master Control Bus and all QBus Hardware

- Replace the MCC interface panels.

- Forth Software

- TCS support daemons and application software in IRTF workstations as required.

Should we explicitly add the manual mode here?

1.2 Sky Coverage

Dec Range is -53:05 to +62:07 (deg:min)

HA Range is -05:04 to + 05:04 (hr:min)

1.3 Pointing Accuracy

Slew to an object up to 10 degrees from a known object to +- 2 arc sec (within a cone of 60 degrees from zenith).

Slew to any point to +- 10 arc sec (within a cone of 60 degrees from zenith).

Agree with Jim’s removal of this statement

1.4 Tracking

+- 2 arc sec per hour accuracy within ( a core of 0.5 hrs from zenith).

Short term (10 mins) RMS error of +- 0.1 (within a cone of 60 degrees from zenith).

Maximum allowed speed is 400 arc sec/sec.

Maximum acceleration is 400 arcsec/sec2.

Non-Sidereal (Base) rate limited to <= +- 100 arcsec/sec.

What is a fastest non sidereal rate requested of the IRTF?

We should add a track adjustment resolution spec to make it easier to flowdown  controller requirements, e.g. 0.01 arcsec/sec.

Also we may need to include a wind buffeting (or max wind speed) spec for operation (partially defines stiffness requirement).

1.5 Offsetting and Scanning

Accuracy of <= 1 arc sec for offset less that 1 degree.

For a 10 arcsec move: Total Time = 1 sec, Damping Time = 0.5 sec.

For a 2 arcmin move: Total Time = 2.5 sec, Damping Time = 0.5 sec.

Possibly add feedforward mode specs here?

Individual offset limit are +-600 arc sec (10 minutes).

Scan rate +- 100 arcsec/sec.

Are we intending to have a spiral or raster scan mode?

1.6 Slewing

Maximum speed is 2025 as/s (HA) and 1620 as/s (DEC).

Maximum acceleration is 1200 arcsec/sec2.

Slew speeds enabled only while operator is actively depressing slew enable switch.

When not depressed =?

1.7 Implement flexible design to allow for future enhancements. List of possible upgrades:

Remote operations by the Telescope Operator may be desired in the future. This system should be flexible enough to allow modifications for remote operation, if necessary.

Possible 1.8 section for  known structural resonances for telescope and secondary rings….

2. Computer Hardware Requirements

Equipment Rack - 19” Equipment Rack, 4 post open frame, 30”D x 80.1” H (43U), with wheels.3 fixed shelves, 3 sliding shelves. Include AC Power distribution.

Network Switch – Each rack contains a 12 port, 10/100 switch.

Computer System – Computer system will run the TCS3 software.

- 3 computers systems (Summit, summit spare, lab system).

- All computer system identical in configuration.

- 4 computer monitors (2 for summit, 2 for lab).

- Linux compatible hardware

- Can accommodate either PMAC or DMC 18x0 PCI motor controller.

- Computer configured for Dual Video

- Dual boot with Windows XP and Linux.

Maybe explicitly add ups power capability (not just building UPS).

Servo Controller – Used on control the HA, Dec, and Dome axis.

- PCI format

- Position range: >1,296,000,000 counts

- Able to interface to T3 custom electronics

- 4 axis of control (HA, Dec, Dome will be targeted).

- Modes of motion: point-to-point, jog

- PID filter

Need to add feedfoward mode and tachometer loops.

- 16-bit DAC output

- Inputs from increment encoders and rate tachometers for each axis

- Limit switch inputs.

Lab Motor- A Lab Motor setup is needed to test motor control hardware and software in the lab.

- 2 units (axis) are required to allow testing for HA, and Dec control.

- 2 motors per unit (axis) to mimic the IRTF opposing torque (anti-backlash) design.

Add tachometer per motor.

- Torque/Current Control servo amplifiers.

- Motor RPM spec: 1200 as/s * 144/1296000 as/rev = 0.13 rps

- Motor-axis spec: 144:1 gear ratio or 9000 arcsec/rev (same as IRTF)

- Increment encoder resolution: 100 counts/arc sec at the load output shaft.

- Design should accommodate up to 40 lbs on axis to provide inertia.

- Implement an adjustable inertia feature; ideally mimic the torque/weight ratio of IRTF.

- Implement an adjustable spring constant mechanism.

- Implement an adjustable friction drag mechanism.

- Provide slew, stop, brake limits switch (Dry contact).

3. T3 Electronics

- DEC and HA axis are split into two voltage ranges, -10V to 0 and 0 to +10V which is output to the respective power amplifiers.

What is the source or intent of this statement????

- An adjustable preload shall be output to each power amplifier.

We should be using one controller axis per motor.

- Voltage output to the DEC and HA amplifiers shall be reduced to a pre-determined level when a slew switch is tripped.

Just semantics – is this for the slew enable or is this an overspeed circuit trip?

- DEC, HA, Dome axis set the voltage output to the power amplifiers to 0 when an emergency or brake lockout condition is active.

- Emergency or brake lockout condition can be overridden via a key switch from the TO panel.

- An emergency condition is defined as follows:

  - Brake limit switch is tripped.

We will need to explicity define the conditions for a trip in a separate safety document. Good start though…

  - Amplifier overload monitor output is active.

  - Emergency stop button is tripped.

  - Facility I/O emergency output is active.

  - Watchdog timer that monitors the TCS computer times out.

  - 2025 as/s over speed condition originating from the motor tachometer.

- A brake lockout condition is defined as follows:

  - Lockout output from the TCS computer.

  - Key switch is in the lockout position.

  - Error output originating from the motor controller card.

- Control of the 115V brake relay is based on the emergency and brake lockout condition.

- The 115V brake relay is enabled via the brake reset switch located on the TO panel.

- The LED indicators present on the TO panel are as follows:

  - Brake On

  - Emergency

  - North Slew

  - North Stop

  - South Slew

  - South Stop

  - East Slew

  - East Stop

  - West Slew

  - West Stop

  - Zenith

  - Lockout

  - Standard

  - Horizon Slew

  - Horizon Stop

4. Encoders

Absolute position resolution: 3.0 arcsec	[>6.0 arcsec  is current TCS resolution]

Incremental position resolution: 0.01 arcsec	[0.2 arcsec is current TCS resolution]

Maximum slew rate: 2025 arcsec/sec on sky

Maximum slew acceleration: 1200 arcsec/sec2

Environment:	 -10 to 50 deg C.

Humidity:	20%-100% Relative Humidity.

5.1 MCC Replacement

Provide 2 LCD display for TCS control and display. Referred to as Display1 and Display2.

Provide 1 Hardware Panel (referred to as the TO Panel) for TCS control and display.

There devices are to be duplicated at the Hilo lab configuration.

Display1/2 and TO Panel will be in continuous development over the course of the lab development phase. These requirements give the project an initial prototype target. Staff input will allow the displays to evolve through development.

Refreshed GUI at 10 Hz.

5.1.1 Display1 Requirements

Show sidereal time, UTC and Local Time

Show RA, DEC and HA in Target, Mean, Increment Encoder Unit, and Abs Encoder Units.

Show stop/slew/brake limits; zenith indicator; horizontal limits; Emergency indicator; lockout indicator.

Provide shutter control widgets.

Provide Dome control widgets.

Indicated T3 ServoMode with performance data.

Provide various warning indicators.

Provide widgets for Mirror Cover control

Provide widgets for Mirror Support control.

Provide widgets for HA, Dec brake control.

Provide warning indicator.

5.1.2 Display2 Requirements

Display HA motor currents (E, W), and differential current for each axis.

Display Dec motor currents (N, S), and differential current for each axis.

Display Dome motor current (3 motors).

Provide widget to enable/disable dome CW control.

Provide widget to control and view position of the dome counterweight (10 items).

Be able to save a counter weight configuration.

Be able to restore a saved counter weight configuration.

5.1.3 TO Panel Requirements

Form Factor approx 14W x 15H x 8D.

30 ft. cable from T3 Electronic rack.

Provides direct hardware control directly to T3 Electronics

Computer Control (ON=computer doing servo, OFF=brake lock condition).

Limit Override ( ON=ignore limits; OFF=use limit inputs)

Emergency Stop (Push to Enable Emergency)

Emergency Reset (To Display emergency)

Telescope Brake Control ( Switch + Enable button; LED to indicate ON).

LED to indicate Slew, Stop, Brake Limit for N, S, E, W. (3 x 4 = 12 inputs)

LED to indicate Zenith

LED to indicate Horizon Limit (2 TCS inputs)

LED to indicate Emergency condition on Safety Circuit

LED to indicated BrakeLockOut condition on Safety Circuit

6. Facility IO

The bulk of the IO signal coming into the current MCC will be replaced with a offset the shelf IO hardware. The following general requirements are established.

- 10 Hz update rates

- Additional IO can be easily added.

- Widely used in Industrial Automation

- Wide variety of the IO options.

- Ethernet based interface

7. Software

- Maintain position table. Data item for position table to evolve per TO and scientific staff input. Basic information includes:

  - RA, Dec base position in Time, Degrees

  - RA,  Dec rate in as/s

  - 2 user offset (beam, user) in as/s. Total offset limited to 600 as/s.

  - scan offset to include RA, Dec offset, and desired rate( as/s)

- Calculate Target (RA, Dec) to mount (HA, Dec) coordinates at minimum rate of 10 Hz.

- Monitor and set Facility IO hardware at a minimum rate of 10 Hz.

- Pointing Correction

  - Develop procedure to collect pointing data for TPOINT utility.

  - Apply TPOINT pointing corrections for equatorial mount.

  - Allow user modification of TPOINT IH and ID coefficients in real-time.

- Provide network based communication for instrumentation and guiders using TCP/IP.

- Provide source code for visitor instrumentations that allow socket communication to TCS. Visitor computer must support the Berkley socket API (All UNIX-like systems, and windows after Win98 support this API).

- Develop GUI to display similar data item provide by TCS1 MCC. Data item and format to evolve per TO and staff input.

- Develop Remote observation GUI for remove observers similar to tcs1_status.

- Develop a TO GUI interface.

- Allow any GUI to be remotely exported using the internet.

- TCS mode for servo should include: tracking, slewing, direct motor position, and Jog.

- Diagnostics and Utilities

  - Procedure to calculate Encoder Rations.

- Safety Related

  - Trigger Safety Circuit Watch Dog Timer at 10 Hz.

  - Monitor ABS vs INC. Enable BrakeLock on  ABS-INC > 2 deg.

  - Monitor Speed. Enable BrakeLock on speed > 25000 as/s.

  - Enable BrakeLock for ServoPosExceedLimit (Commanded vs Encoder position) > 2 deg.

  - Software HA and Dec Limits are default to SLEW Limits, but can be adjust up to STOP Limit. Software limits can be disabled by tcs3 command.

Will need to add software and network security measures in the safety document.

Revision History

Sept 2003

Max Slew rate adjusted to 2025as/s (DEC) and 1620 as/s (HA). Based on DayCrew measurements. These values should be adjusted.

Document Sign Off

The requirement specifications contained in this document have been reviewed and agreed upon.

Tony Denault, T3 Project Manager		_________________________________

Alan Tokunaga, IRTF Division Chief	_________________________________

Peter Onaka, IRTF Chief Engineer		_________________________________

George Koenig, IRTF Superintendent	_________________________________